การทดลองที่ 10 การใช้งาน RTC DS3231

กลุ่มที่ 4

1.นางสาว พิมพ์พิมล ราชหิรินทร์ 66172110035-4

2. นาย กีรติ ชื่นจอหอ 66172110370-2

3. นาย จิรวัฒน์ เทียมทะนงค์ 66172110399-8

วัตถุประสงค์

1. ศึกษาการทำงานของไอซี RTC DS3231

2. ศึกษาการเขียนโปรแกรมควบคุมการทำงานไอซี RTC DS3231

ขั้นตอนการทดลอง

1. ให้นักศึกษาต่อวงจรเชื่อมต่อบอร์ด ESP32 ต่อไปนี้

2. เขียนโปรแกรมควบคุมดีซีมอเตอร์ โดยให้กำหนดการทำงานของมอเตอร์ผ่านทางคีย์แพ็ค มอเตอร์หมุนตามเวลาเปิด/ปิดต่อขา D2 เข้ากับขา OPA ของโมดูลมอเตอร์ นับจำนวนรอบที่มอเตอร์หมุนโดยการอินเตอร์รัพท์ และแสดงผลการทำงานผ่านทางโมดูล I2C LCD ซึ่งโปรแกรมมีการทำงานดังตาราง

| ป้อนข้อมูลผ่านคีย์แพ็ด | การทำงาน |
| --- | --- |
| 0 | ตั้งเวลาปัจจุบัน |
| 1 | หยุดหมุน |
| 2 | กำหนดให้ทิศทางหมุนไปทางขวา |
| 3 | กำหนดให้ทิศทางหมุนไปทางซ้าย |
| 4 | ตั้งค่าความเร็วในการหมุน |
| 5 | สั่งให้มอเตอร์หมุน |
| 6 | ตั้งเวลาเริ่มต้นในการหมุนของมอเตอร์ |
| 7 | ตั้งเวลาสิ้นสุดในการหมุนของมอเตอร์ |

หมายเหตุ:

1. การป้อนความเร็วในการหมุนของมอเตอร์ให้ป้อนค่าความเร็วหน่วยเป็น เปอร์เซนต์และหลังจากป้อนเสร็จแล้วให้กดปุ่ม ‘#’

2. การป้อนเวลาเริ่มและหยุดหมุนให้ป้อนเริ่มจากหน่วยชั่วโมง นาที และวินาทีตามลำดับ หลังป้อนเวลาแต่ละหน่วยเสร็จให้กดปุ่ม ‘#’

2.1 เขียนโฟลว์ชาร์ต

โฟลว์ชาร์ตโปรแกรมหลัก

โฟลว์ชาร์ตของฟังก์ชัน

โฟลว์ชาร์ตของฟังก์ชัน

โฟลว์ชาร์ตของฟังก์ชัน

2.2 เขียนโปรแกรม

from machine import Pin, PWM, I2C

from time import sleep, ticks_ms

from sys import exit

from lcd_i2c import I2cLcd

from ds3231 import DS3231

# --- Hardware & Keypad Configuration ---

KEY_MATRIX = [

['1', '2', '3', 'A'],

['4', '5', '6', 'B'],

['7', '8', '9', 'C'],

['*', '0', '#', 'D']

]

COLUMN_PINS = [26, 27, 32, 33]

ROW_PINS = [12, 13, 14, 15]

# --- Hardware Initialization ---

columns = [Pin(pin, Pin.IN, Pin.PULL_DOWN) for pin in COLUMN_PINS]

rows = [Pin(pin, Pin.OUT) for pin in ROW_PINS]

# Motor Driver Pins

EN_pin = Pin(5, Pin.OUT)

PWM1 = PWM(Pin(18), freq=5000)

PWM2 = PWM(Pin(19), freq=5000)

# I2C Devices Initialization

i2c = I2C(0, sda=Pin(21), scl=Pin(22), freq=200000)

lcd = I2cLcd(i2c, i2c.scan()[0], 2, 16)

rtc = DS3231(i2c)

# --- Global State Dictionary (เหมือนเดิม) ---

data = {

"status": "Stop", "direction": 0, "speed": 0,

"percen_speed": 0, "last_time": 0, "count": 0,

"start_time": 0, "end_time": 0, "state": 0

}

# --- Function Definitions (เปลี่ยนเฉพาะชื่อ) ---

def spin_motor_left(speed_duty):

EN_pin.value(1)

PWM1.duty(0)

PWM2.duty(int(speed_duty))

def spin_motor_right(speed_duty):

EN_pin.value(1)

PWM1.duty(int(speed_duty))

PWM2.duty(0)

def stop_motor():

EN_pin.value(0)

PWM1.duty(0)

PWM2.duty(0)

def start_motor_by_direction(direction):

if direction == -1:

spin_motor_left(data["speed"])

data["status"] = "Left"

elif direction == 1:

spin_motor_right(data["speed"])

data["status"] = "Right"

def scan_keypad():

for row in range(len(rows)):

rows[row].value(1)

for col in range(len(columns)):

if columns[col].value() == 1:

key = KEY_MATRIX[row][col]

rows[row].value(0)

return key

rows[row].value(0)

return ""

def get_number_from_keypad():

while scan_keypad(): sleep(0.01)

num = ""

while True:

key = scan_keypad()

if key.isdigit() and len(num) <= 3:

num += key

lcd.putstr(key)

elif (key == '#') and num:

return int(num)

while scan_keypad(): sleep(0.01)

sleep(0.01)

def handle_speed_setting_flow():

lcd.clear()

lcd.putstr("Set (1-100): ")

number = get_number_from_keypad()

if number > 100:

lcd.clear()

lcd.putstr("Out of range.")

lcd.move_to(0, 1)

lcd.putstr("press '*' ")

while scan_keypad() != '*':

sleep(0.01)

else:

data["speed"] = (7.23 * number) + 300

data["percen_speed"] = number

if data["status"] == "Right" and data["state"]:

spin_motor_right(data["speed"])

elif data["status"] == "Left" and data["state"]:

spin_motor_left(data["speed"])

def get_time_component_from_keypad():

while scan_keypad(): sleep(0.01)

num = ""

lcd.show_cursor()

lcd.blink_cursor_on()

while True:

key = scan_keypad()

if key.isdigit() and len(num) <= 1:

num += key

lcd.putstr(key)

if len(num) == 2:

lcd.hide_cursor()

elif (key == '#') and num:

return int(num)

while scan_keypad(): sleep(0.01)

sleep(0.01)

def clear_lcd_area(x, y, amount, char=' '):

lcd.move_to(x, y)

for _ in range(amount):

lcd.putstr(char)

lcd.move_to(x, y)

def get_full_time_from_keypad():

while scan_keypad(): sleep(0.01)

lcd.move_to(4, 1)

hour = get_time_component_from_keypad()

while hour > 23:

clear_lcd_area(4, 1, 2, '0')

hour = get_time_component_from_keypad()

lcd.move_to(7, 1)

minute = get_time_component_from_keypad()

while minute > 59:

clear_lcd_area(7, 1, 2, '0')

minute = get_time_component_from_keypad()

lcd.move_to(10, 1)

second = get_time_component_from_keypad()

while second > 59:

clear_lcd_area(10, 1, 2, '0')

second = get_time_component_from_keypad()

lcd.hide_cursor()

lcd.clear()

return hour, minute, second

def handle_set_current_time_flow():

lcd.clear()

lcd.putstr("   Set time")

lcd.move_to(1, 1)

lcd.putstr(">> 00:00:00 <<")

hour, minute, second = get_full_time_from_keypad()

current = rtc.datetime()

new_date = [current[0], current[1], current[2], hour, minute, second, 1]

rtc.datetime(new_date)

lcd.hide_cursor()

lcd.clear()

def handle_set_schedule_time_flow(mode): # 0 for start, 1 for end

current = rtc.datetime()

lcd.clear()

if mode == 0:

lcd.putstr("   Start time") cvbnbcbbcb

else:

lcd.putstr("    End time")

lcd.move_to(1, 1)

lcd.putstr(f">> {current[4]:02}:{current[5]:02}:{current[6]:02} <<")

hour, minute, second = get_full_time_from_keypad()

total_seconds = (hour * 3600) + (minute * 60) + second

if mode == 0:

data["start_time"] = total_seconds

else:

data["end_time"] = total_seconds

def map_key_to_command(value):

if value == '0': return "set_current"

if value == '1': return "off"

if value == '2': return "right"

if value == '3': return "left"

if value == '4': return "set_speed"

if value == '5': return "on"

if value == '6': return "set_start"

if value == '7': return "set_end"

return ""

def execute_command(command):

state = data["state"]

speed = data["speed"]

if command == "set_current":

handle_set_current_time_flow()

elif command == "off":

stop_motor()

data["status"] = "Stop"

data["state"] = 0

elif command == "right":

data["direction"] = 1

data["count"] = 0

data["status"] = "Right"

if state: spin_motor_right(speed)

elif command == "left":

data["direction"] = -1

data["count"] = 0

data["status"] = "Left"

if state: spin_motor_left(speed)

elif command == "set_speed":

handle_speed_setting_flow()

data["count"] = 0

elif command == "on":

start_motor_by_direction(data["direction"])

data["state"] = 1

elif command == "set_start":

handle_set_schedule_time_flow(0)

elif command == "set_end":

handle_set_schedule_time_flow(1)

display_main_status()

def display_main_status():

current = rtc.datetime()

lcd.clear()

lcd.putstr(f"{current[4]:02}:{current[5]:02}:{current[6]:02} {data['status']}")

lcd.move_to(0, 1)

lcd.putstr(f"C:{data['count']//20}")

lcd.move_to(9, 1)

lcd.putstr(f"S:{data['percen_speed']}%")

def handle_encoder_interrupt(pin):

new_time = ticks_ms()

if (new_time - data["last_time"]) > 2:

data["count"] += 1

data["last_time"] = new_time

def update_runtime_display():

current = rtc.datetime()

lcd.move_to(0, 0)

lcd.putstr(f"{current[4]:02}:{current[5]:02}:{current[6]:02}")

lcd.move_to(3, 1)

lcd.putstr(f"{data['count']//20}")

def check_and_run_schedule():

current = rtc.datetime()

current_time_sec = (current[4] * 3600) + (current[5] * 60) + current[6]

start = data["start_time"]

end = data["end_time"]

if end > start and start <= current_time_sec <= end and data["direction"] != 0:

start_motor_by_direction(data["direction"])

data["count"] = 0

display_main_status()

# This while loop blocks the main loop

while current_time_sec <= end:

update_runtime_display()

sleep(1)

current = rtc.datetime()

current_time_sec = (current[4] * 3600) + (current[5] * 60) + current[6]

stop_motor()

data["status"] = "Stop"

data["start_time"] = 0

data["end_time"] = 0

display_main_status()

else:

update_runtime_display()

# --- Interrupt Setup ---

interr_pin = Pin(2, Pin.IN)

interr_pin.irq(trigger=Pin.IRQ_FALLING, handler=handle_encoder_interrupt)

# --- Main Program Loop ---

if __name__ == "__main__":

display_main_status()

while True:

key = scan_keypad()

op = map_key_to_command(key)

if op:

execute_command(op)

check_and_run_schedule()

sleep(1)

2.3 ให้นักศึกษาทำการรันโปรแกรมและลงบันทึกผลรันในตารางต่อไปนี้

|  | ป้อนข้อมูลผ่านคีย์แพ็ด | การแสดงผลของแอลซีดี | การทำงานของดีซีมอเตอร์ |
| --- | --- | --- | --- |
| 1 | 0 xx#xx#xx# |  | มอเตอร์หยุดหมุน |
| 2 | 1 |  | มอเตอร์หยุดหมุน |
|  | 2 |  | เปลี่ยนทิศทางของมอเตอร์ไปทางขวา |
| 4 | 3 |  | เปลี่ยนทิศทางของมอเตอร์ไปทางซ้าย |
| 5 | 4 50# |  | เซ็ทสปีดเป็น50% |
| 6 | 5 |  | มอเตอร์หมุนไปทางซ้ายความเร็ว 50% |
| 7 | 4 90# |  | เซ็ทสปีดเป็น90% |
| 8 | 5 |  | มอเตอร์หมุนไปทางซ้ายความเร็ว 90% |
| 9 | 6 xx#xx#xx# |  | ตั้งเวลาเริ่มหมุนเป็น 11:00:00 |
| 10 | 7 xx#xx#xx# |  | ตั้งเวลาสิ้นสุดหมุนเป็น 11:01:00 |
| 12 |  |  |  |

หมายเหตุ: ก่อนรันโปรแกรมให้ต่อขา D2 เข้ากับขา OPA ของโมดูลมอเตอร์