การทดลองที่ 11 การควบคุมรีเลย์

กลุ่มที่ 4

1.นางสาว พิมพ์พิมล ราชหิรินทร์ 66172110035-4

2. นาย กีรติ ชื่นจอหอ 66172110370-2

3. นาย จิรวัฒน์ เทียมทะนงค์ 66172110399-8

วัตถุประสงค์

1. ศึกษาการทำงานของรีเลย์

2. ศึกษาการเขียนโปรแกรมควบคุมการทำงานรีเลย์

ขั้นตอนการทดลอง

1. ให้นักศึกษาต่อวงจรเชื่อมต่อบอร์ด ESP32 ต่อไปนี้

2. เขียนโปรแกรมตั้งเวลาควบคุมอุปกรณ์ไฟฟ้าที่ต่อกับขา D4 โดยกำหนดให้ตั้งเวลาเปิดปิดโดยสวิตช์เมตริกซ์ และแสดงเวลาออกทางโมดูล I2C LCD ซึ่งโปรแกรมมีการทำงานดังตาราง

| ป้อนข้อมูลผ่านคีย์แพ็ด | การทำงาน |
| --- | --- |
| 1 | ตั้งเวลาปัจจุบัน |
| 2 | แสดงเวลา |
| 3 | อุปกรณ์ไฟฟ้า OFF |
| 4 | อุปกรณ์ไฟฟ้า ON |
| 5 | ไทม์เมอร์ OFF |
| 6 | ไทม์เมอร์ ON |
| 7 | ตั้งเวลาเริ่มต้นการทำงานของอุปกรณ์ไฟฟ้า |
| 8 | ตั้งเวลาสิ้นสุดการทำงานของอุปกรณ์ไฟฟ้า |

หมายเหตุ:

1. การป้อนเวลาให้ป้อนเริ่มจากหน่วยชั่วโมง นาที และวินาทีตามลำดับ หลังป้อนเวลาแต่ละหน่วยเสร็จให้กดปุ่ม ‘#’

2.1 เขียนโฟลว์ชาร์ต

โฟลว์ชาร์ตโปรแกรมหลัก

โฟลว์ชาร์ตของฟังก์ชัน

โฟลว์ชาร์ตของฟังก์ชัน

โฟลว์ชาร์ตของฟังก์ชัน

โฟลว์ชาร์ตของฟังก์ชัน

โฟลว์ชาร์ตของฟังก์ชัน

โฟลว์ชาร์ตของฟังก์ชัน

2.2 เขียนโปรแกรม

from machine import Pin, I2C

from time import sleep

from lcd_i2c import I2cLcd

from ds3231 import DS3231

# --- Hardware & Keypad Configuration ---

KEY_MATRIX = [

['1', '2', '3', 'A'],

['4', '5', '6', 'B'],

['7', '8', '9', 'C'],

['*', '0', '#', 'D']

]

COLUMN_PINS = [26, 27, 32, 33]

ROW_PINS = [12, 13, 14, 15]

# --- Hardware Initialization ---

columns = [Pin(pin, Pin.IN, Pin.PULL_DOWN) for pin in COLUMN_PINS]

rows = [Pin(pin, Pin.OUT) for pin in ROW_PINS]

relay = Pin(4, Pin.OUT)

relay.value(0)

i2c = I2C(0, sda=Pin(21), scl=Pin(22), freq=200000)

lcd = I2cLcd(i2c, i2c.scan()[0], 2, 16)

rtc = DS3231(i2c)

# --- Global State Dictionary ---

data = {

"start_time": 0, "end_time": 0, "timer": "Off",

"timer_state": False, "device": "Off", "show_time": False

}

# --- Function Definitions ---

def scan_keypad():

key = ''

for row in range(len(rows)):

rows[row].value(1)

for col in range(len(columns)):

if columns[col].value() == 1:

key = KEY_MATRIX[row][col]

rows[row].value(0)

return key

rows[row].value(0)

return key

def clear_lcd_area(x, y, amount, char=' '):

lcd.move_to(x, y)

for _ in range(amount):

lcd.putstr(char)

lcd.move_to(x, y)

def get_time_component_from_keypad():

while scan_keypad(): sleep(0.01)

num = ""

lcd.show_cursor()

lcd.blink_cursor_on()

while True:

key = scan_keypad()

if key.isdigit() and len(num) < 2:

num += key

lcd.putstr(key)

if len(num) == 2:

lcd.hide_cursor()

elif (key == '#') and num:

return int(num)

while scan_keypad(): sleep(0.01)

sleep(0.02)

def get_full_time_from_keypad():

while scan_keypad(): sleep(0.01)

lcd.move_to(4, 1)

hour = get_time_component_from_keypad()

while hour > 23:

clear_lcd_area(4, 1, 2, '0')

hour = get_time_component_from_keypad()

lcd.move_to(7, 1)

minute = get_time_component_from_keypad()

while minute > 59:

clear_lcd_area(7, 1, 2, '0')

minute = get_time_component_from_keypad()

lcd.move_to(10, 1)

second = get_time_component_from_keypad()

while second > 59:

clear_lcd_area(10, 1, 2, '0')

second = get_time_component_from_keypad()

lcd.hide_cursor()

lcd.clear()

return hour, minute, second

def handle_set_time_flow(mode): # 1=Current, 2=Start, 3=End

current = rtc.datetime()

lcd.clear()

if mode == 1:

lcd.putstr("   Set time")

elif mode == 2:

lcd.putstr("   Start time")

else:

lcd.putstr("    End time")

lcd.move_to(1, 1)

lcd.putstr(f">> {current[4]:02}:{current[5]:02}:{current[6]:02} <<")

hour, minute, second = get_full_time_from_keypad()

if mode == 1:

new_date = [current[0], current[1], current[2], hour, minute, second, current[3]]

rtc.datetime(new_date)

elif mode == 2:

data["start_time"] = (hour * 3600) + (minute * 60) + second

else:

data["end_time"] = (hour * 3600) + (minute * 60) + second

def update_display():

if data['show_time']:

current = rtc.datetime()

clear_lcd_area(0, 0, 4)

lcd.move_to(4, 0)

lcd.putstr(f"{current[4]:02}:{current[5]:02}:{current[6]:02}")

lcd.move_to(0, 1)

lcd.putstr(f"TMR:{data['timer']} DEV:{data['device']}  ")

else:

lcd.move_to(0, 0)

lcd.putstr(f"Timer: {data['timer']} ")

clear_lcd_area(10, 0, 6)

lcd.move_to(0, 1)

lcd.putstr(f"Device: {data['device']} ")

clear_lcd_area(11, 1, 5)

def set_device_state(is_on):

if is_on:

relay.value(is_on)

data['device'] = "On"

else:

relay.value(is_on)

data['device'] = "Off"

def map_key_to_command(value):

operate_map = {

'1': "set_current", '2': "toggle_time_view",

'3': "device_off", '4': "device_on",

'5': "timer_off", '6': "timer_on",

'7': "set_start", '8': "set_end"

}

return operate_map.get(str(value), "")

def execute_command(command):

if command == "set_current":

handle_set_time_flow(1)

elif command == "toggle_time_view":

data['show_time'] = not data['show_time']

elif command == "device_off":

set_device_state(0)

elif command == "device_on":

set_device_state(1)

elif command == "timer_off":

data['timer'] = "Off"

elif command == "timer_on":

data['timer'] = "On"

elif command == "set_start":

handle_set_time_flow(2)

elif command == "set_end":

handle_set_time_flow(3)

def check_and_run_timer():

current = rtc.datetime()

current_time_sec = (current[4] * 3600) + (current[5] * 60) + current[6]

if data['start_time'] <= current_time_sec <= data['end_time'] and not data['timer_state']:

data['timer_state'] = True

set_device_state(1)

elif current_time_sec > data['end_time'] and data['timer_state']:

data['timer_state'] = False

set_device_state(0)

# --- Main Program Loop ---

if __name__ == "__main__":

while True:

key = scan_keypad()

if key:

op = map_key_to_command(key)

if op:

execute_command(op)

if data['timer'] == "On":

check_and_run_timer()

update_display()

sleep(0.02)

2.3 ให้นักศึกษาทำการรันโปรแกรมและลงบันทึกผลรันในตารางต่อไปนี้

| ครั้งที่รันโปรแกรม | ป้อนข้อมูลผ่านคีย์แพ็ด | การแสดงผลของแอลซีดี | การทำงานของอุปกรณ์ไฟฟ้า |
| --- | --- | --- | --- |
| 1 | 0 xx#xx#xx# | ไม่มีอะไรเกิดขึ้น | ไม่มีอะไรเกิดขึ้น |
|  | 1 |  | เซ็ทเวลา 10:00:00 |
|  | 2 |  | ดูเวลาที่เซ็ทไว้ |
|  | 3 |  | อุปกรณ์ไฟฟ้า OFF |
| 5 | 2 |  | ดูเวลา |
|  | 4 |  | อุปกรณ์ไฟฟ้า ON |
| 7 | 7 xx#xx#xx# |  | เวลาเริ่มที่ดีเลย์ที่ LED ติด |
| 8 | 8 xx#xx#xx# |  | เวลาสิ้นสุดที่ดีเลย์ที่ LED ดับ |
|  | 5 |  | ไทม์เมอร์ OFF |
| 10 | 3 |  | อุปกรณ์ไฟฟ้า OFF |
| 11 | 2 |  | เวลาสิ้นสุดที่ดีเลย์ที่ LED ดับ |