การทดลองที่ 13 การใช้งาน DHT22

กลุ่มที่ 4

1.นางสาว พิมพ์พิมล ราชหิรินทร์ 66172110035-4

2. นาย กีรติ ชื่นจอหอ 66172110370-2

3. นาย จิรวัฒน์ เทียมทะนงค์ 66172110399-8

วัตถุประสงค์

1. ศึกษาการทำงานของ DHT22

2. ศึกษาการเขียนโปรแกรมอ่านอุณหภูมิและความชื้นอากาศจาก DHT22

ขั้นตอนการทดลอง

1. ให้นักศึกษาต่อวงจรเชื่อมต่อบอร์ด ESP32 ต่อไปนี้

1. เครื่องควบคุมอุปกรณ์ไฟฟ้าตามอุณหภูมิแบบตั้งเวลามีฟังก์ชันการทำงานแสดงดังตาราง

- สามารถตั้งเวลาเปิดปิดได้อย่างน้อย 2 ช่วงเวลา

- สามารถกำหนดช่วงค่าอุณหภูมิที่ต้องการควบคุมได้

- ข้อมูลที่ใช้ในการควบคุมและสถานะการทำงานของอุปกรณ์จะไม่สูญหาย เมื่อมีการปิด-เปิดหรือรีเซ็ตเครื่อง

2. เขียนโปรแกรมตั้งเวลาควบคุมอุปกรณ์ไฟฟ้าที่ต่อกับขา D27 โดยกำหนดให้ตั้งเวลาและอุณหภูมิเปิดปิดโดยสวิตช์เมตริกซ์ และแสดงเวลาออกทางโมดูล I2C LCD ซึ่งโปรแกรมมีการทำงานดังตาราง

| ป้อนข้อมูลผ่านคีย์แพ็ด | การทำงาน |
| --- | --- |
| 0 | ตั้งเวลาปัจจุบัน |
| 1 | แสดงเวลาปัจจุบัน |
| 2 | อุปกรณ์ไฟฟ้า OFF |
| 3 | อุปกรณ์ไฟฟ้า ON |
| 4 | ไทม์เมอร์ OFF |
| 5 | ไทม์เมอร์ ON |
| 6 | ตั้งเวลาเริ่มต้นการทำงานของอุปกรณ์ไฟฟ้า |
| 7 | ตั้งเวลาสิ้นสุดการทำงานของอุปกรณ์ไฟฟ้า |
| 8 | แสดงเวลาเริ่มต้นการทำงานของอุปกรณ์ไฟฟ้า |
| 9 | แสดงเวลาสิ้นสุดการทำงานของอุปกรณ์ไฟฟ้า |
| A | ตั้งอุณหภูมิเริ่มต้นการทำงานของอุปกรณ์ไฟฟ้า |
| B | ตั้งอุณหภูมิสิ้นสุดการทำงานของอุปกรณ์ไฟฟ้า |
| C | แสดงอุณหภูมิเริ่มต้นการทำงานของอุปกรณ์ไฟฟ้า |
| D | แสดงอุณหภูมิสิ้นสุดการทำงานของอุปกรณ์ไฟฟ้า |

หมายเหตุ:

1. การป้อนเวลาให้ป้อนเริ่มจากหน่วยชั่วโมง นาที และวินาทีตามลำดับ หลังป้อนเวลาแต่ละหน่วยเสร็จให้กดปุ่ม ‘#’

2.1 เขียนโฟลว์ชาร์ต

โฟลว์ชาร์ตโปรแกรมหลัก

โฟลว์ชาร์ตของฟังก์ชัน

โฟลว์ชาร์ตของฟังก์ชัน

โฟลว์ชาร์ตของฟังก์ชัน

โฟลว์ชาร์ตของฟังก์ชัน

โฟลว์ชาร์ตของฟังก์ชัน

2.2 เขียนโปรแกรม

from machine import Pin, I2C

from time import sleep

from lcd_i2c import I2cLcd

from ds3231 import DS3231

import json

import dht

# --- Hardware & Keypad Configuration ---

KEY_MATRIX = [

['1', '2', '3', 'A'],

['4', '5', '6', 'B'],

['7', '8', '9', 'C'],

['*', '0', '#', 'D']

]

COLUMN_PINS = [26, 27, 32, 33]

ROW_PINS = [12, 13, 14, 15]

# --- Hardware Initialization ---

columns = [Pin(pin, Pin.IN, Pin.PULL_DOWN) for pin in COLUMN_PINS]

rows = [Pin(pin, Pin.OUT) for pin in ROW_PINS]

relay = Pin(4, Pin.OUT)

relay.value(0)

i2c = I2C(0, sda=Pin(21), scl=Pin(22), freq=200000)

lcd = I2cLcd(i2c, i2c.scan()[0], 2, 16)

rtc = DS3231(i2c)

sensor = dht.DHT22(Pin(25))

# --- Global State Dictionary ---

data = {

"start_time": 0, "end_time": 0, "start_temp": 25, "end_temp": 35,

"timer": "Off", "device": "Off", "time_mode": 0, "temp_mode": 0

}

# --- Function Definitions ---

def scan_keypad():

key = ''

for row in range(len(rows)):

rows[row].value(1)

for col in range(len(columns)):

if columns[col].value() == 1:

key = KEY_MATRIX[row][col]

rows[row].value(0)

return key

rows[row].value(0)

return key

def clear_lcd_area(x, y, amount, char=' '):

lcd.move_to(x, y)

for _ in range(amount):

lcd.putstr(char)

lcd.move_to(x, y)

def get_generic_number_from_keypad():

while scan_keypad(): sleep(0.01)

num = ""

while True:

key = scan_keypad()

if key.isdigit():

num += key

lcd.putstr(key)

elif (key == '#') and num:

return int(num)

while scan_keypad(): sleep(0.01)

sleep(0.02)

def get_time_component_from_keypad():

while scan_keypad(): sleep(0.01)

num = ""

lcd.show_cursor()

lcd.blink_cursor_on()

while True:

key = scan_keypad()

if key.isdigit() and len(num) < 2:

num += key

lcd.putstr(key)

if len(num) == 2:

lcd.hide_cursor()

elif (key == '#') and num:

return int(num)

while scan_keypad(): sleep(0.01)

sleep(0.02)

def get_full_time_from_keypad():

while scan_keypad(): sleep(0.01)

lcd.move_to(4, 1)

hour = get_time_component_from_keypad()

while hour > 23:

clear_lcd_area(4, 1, 2, '0')

hour = get_time_component_from_keypad()

lcd.move_to(7, 1)

minute = get_time_component_from_keypad()

while minute > 59:

clear_lcd_area(7, 1, 2, '0')

minute = get_time_component_from_keypad()

lcd.move_to(10, 1)

second = get_time_component_from_keypad()

while second > 59:

clear_lcd_area(10, 1, 2, '0')

second = get_time_component_from_keypad()

lcd.hide_cursor()

lcd.clear()

return hour, minute, second

def handle_set_time_flow(mode):

current = rtc.datetime()

lcd.clear()

if mode == 1:

lcd.putstr("   Set time")

elif mode == 2:

lcd.putstr("   Start time")

else:

lcd.putstr("    End time")

lcd.move_to(1, 1)

lcd.putstr(f">> {current[4]:02}:{current[5]:02}:{current[6]:02} <<")

hour, minute, second = get_full_time_from_keypad()

if mode == 1:

new_date = [current[0], current[1], current[2], hour, minute, second, current[3]]

rtc.datetime(new_date)

elif mode == 2:

data["start_time"] = (hour * 3600) + (minute * 60) + second

else:

data["end_time"] = (hour * 3600) + (minute * 60) + second

def handle_set_temperature_flow(mode):

lcd.clear()

if mode == 1:

lcd.putstr("Start Temp: ")

temp = get_generic_number_from_keypad()

data['start_temp'] = temp

else:

lcd.putstr("End Temp: ")

temp = get_generic_number_from_keypad()

data['end_temp'] = temp

def convert_seconds_to_hms(mode):

time_value = 0

if mode == 1:

time_value = data['start_time']

else:

time_value = data['end_time']

hour = time_value // 3600

minute = (time_value % 3600) // 60

second = time_value % 60

return hour, minute, second

def display_selected_time():

if data['time_mode'] == 1:

current = rtc.datetime()

lcd.putstr(f"N:{current[4]:02}:{current[5]:02}:{current[6]:02} ")

elif data['time_mode'] == 2:

hour, minute, second = convert_seconds_to_hms(1)

lcd.putstr(f"S:{hour:02}:{minute:02}:{second:02} ")

else:

hour, minute, second = convert_seconds_to_hms(2)

lcd.putstr(f"E:{hour:02}:{minute:02}:{second:02} ")

def display_temperature_info():

lcd.move_to(11, 0)

lcd.putstr(f"{sensor.temperature()}C")

lcd.move_to(12, 1)

if data['temp_mode'] == 1:

lcd.putstr(f"{data['start_temp']}C")

elif data['temp_mode'] == 2:

lcd.putstr(f"{data['end_temp']}C")

def update_display():

if data['time_mode'] != 0:

lcd.move_to(0, 0)

display_selected_time()

lcd.move_to(0, 1)

lcd.putstr(f"T:{data['timer']} D:{data['device']} ")

else:

lcd.move_to(0, 0)

lcd.putstr(f"Timer: {data['timer']} ")

lcd.move_to(0, 1)

lcd.putstr(f"Device: {data['device']} ")

clear_lcd_area(11, 1, 5)

display_temperature_info()

def set_device_state(is_on):

relay.value(1 if is_on else 0)

data['device'] = "On" if is_on else "Off"

def map_key_to_command(value):

operate_map = {

'0': "set_current", '1': "current_time", '2': "device_off",

'3': "device_on", '4': "timer_off", '5': "timer_on",

'6': "set_start", '7': "set_end", '8': "start_time",

'9': "end_time", 'A': "set_start_temp", 'B': "set_end_temp",

'C': "start_temp", 'D': "end_temp"

}

operate = operate_map.get(str(value), "")

return operate

def execute_command(command):

if command == "set_current": handle_set_time_flow(1)

elif command == "current_time": data['time_mode'] = 1

elif command == "device_off": set_device_state(0)

elif command == "device_on": set_device_state(1)

elif command == "timer_on": data['timer'] = "On"

elif command == "timer_off": data['timer'] = "Off"

elif command == "set_start": handle_set_time_flow(2)

elif command == "set_end": handle_set_time_flow(3)

elif command == "start_time": data['time_mode'] = 2

elif command == "end_time": data['time_mode'] = 3

elif command == "set_start_temp": handle_set_temperature_flow(1)

elif command == "set_end_temp": handle_set_temperature_flow(2)

elif command == "start_temp": data['temp_mode'] = 1

elif command == "end_temp": data['temp_mode'] = 2

save_state_to_file()

def check_and_run_controller():

current = rtc.datetime()

current_time = (current[4] * 3600) + (current[5] * 60) + current[6]

sensor.measure()

temp = sensor.temperature()

if data['timer'] == "On":

if data['start_time'] <= current_time < data['end_time']:

if data['start_temp'] <= temp < data['end_temp'] and data["device"] == "Off":

set_device_state(1)

save_state_to_file()

elif (temp >= data['end_temp'] or temp < data['start_temp']) and data["device"] == "On":

set_device_state(0)

save_state_to_file()

elif current_time >= data['end_time'] and data["device"] == "On":

set_device_state(0)

save_state_to_file()

def load_state_from_file():

global data

try:

with open('backup.json', 'r') as file:

backup_data = json.load(file)

data = backup_data.copy()

set_device_state(1 if data['device'] == "On" else 0)

except:

save_state_to_file()

def save_state_to_file():

with open('backup.json', 'w') as file:

json.dump(data, file)

# --- Main Program Loop ---

if __name__ == "__main__":

load_state_from_file()

while True:

key = scan_keypad()

if key:

op = map_key_to_command(key)

if op:

execute_command(op)

check_and_run_controller()

update_display()

2.3 ให้นักศึกษาทำการรันโปรแกรมและลงบันทึกผลรันในตารางต่อไปนี้

| ครั้งที่รันโปรแกรม | ป้อนข้อมูลผ่านคีย์แพ็ด | การแสดงผลของแอลซีดี | การทำงานของมอเตอร์ดีซี |
| --- | --- | --- | --- |
| 1 | 0 00#00#00# | ตั้งเป็น 0 | ยังไม่การทำงานของอุณหภูมิ และดีเล |
|  | 1 |  | ดูเวลาและอุณหภูมิ |
|  | 3 |  | อุปกรณ์ไฟฟ้า ON |
| 4 | 2 |  | อุปกรณ์ไฟฟ้า OFF |
| 5 | 4 |  | ไทม์เมอร์ OFF |
| 6 | 6 00#01#00# |  | ตั้งเวลาเริ่มต้นการทำงานของอุปกรณ์ไฟฟ้า |
|  | 7 00#03#00# |  | ตั้งเวลาสิ้นสุดการทำงานของอุปกรณ์ไฟฟ้า |
| 8 | 8 |  | แสดงเวลาเริ่มต้นการทำงานของอุปกรณ์ไฟฟ้า |
| 9 | 9 |  | แสดงเวลาสิ้นสุดการทำงานของอุปกรณ์ไฟฟ้า |
|  | A 25# |  | ตั้งอุณหภูมิเริ่มต้นการทำงานของอุปกรณ์ไฟฟ้า |
| 11 | B 35# |  | ตั้งอุณหภูมิสิ้นสุดการทำงานของอุปกรณ์ไฟฟ้า |
|  | C |  | แสดงอุณหภูมิเริ่มต้นการทำงานของอุปกรณ์ไฟฟ้า |
| 13 | D |  | แสดงอุณหภูมิสิ้นสุดการทำงานของอุปกรณ์ไฟฟ้า |
|  | 5 |  | ไทม์เมอร์ ON |
| 15 | กดปุ่ม RESET ของบอร์ด | กลับเข้าสู่หน้าโปรแกรมหลัก | โปรแกรมยังทำงานได้ต่อ |
| 16 | - |  |  |